Designing and Testing a Digital     Front-End Block Using Verilog HDL

The Final Submission

Supervisor: Eng. Eslam Mahmoud

Team 6

Team members

Abdulrahman Saad Samhoud

Ahmed Ibrahim Hassan Ibrahim

Mahmoud Ismail Mahmoud Abdallah

Mohamed Ahmed Mohamed Elsayed

Moheb Mikhael Heshmat

Abbreviation:

| Abbreviation | Description |
| --- | --- |
| IO | Input-Output |
| FD | Fractional Decimator |
| CIC | Cascaded Integrator-Comb |
| IIR | Infinite Impulse Response |
| FIR | Finite Impulse Response |
| LPF | Low Pass Filter |
| fo1 | Interference at 1 MHz |
| fo2 | Interference at 2.4 MHz |
| SNR | Signal to Noise Ratio |
| DUT | Design Under Test |
| AMBA | Advanced Microcontroller Bus Architecture |
| APB | Advanced Peripheral Bus |

Table of Contents

Top Level DFE Overview:	6

System IO Table:	7

DFE IOs:	7

AMBA APB IOs:	7

Stage 1 - Fractional Decimator:	8

The Fractional Decimator Overview:	8

Low pass FIR filter:	8

Down sampling by 3:	8

1.	Hardware Implementation	9

MUX Blocks:	9

2-bit Counter:	9

Multiplier Blocks:	9

Delay Chain:	9

Adder Block:	9

Output Selector:	9

Simulation Diagrams:	10

The Output Result:	10

SNR:	10

Stage 2 – Notch Filter:	11

The Notch Filter Overview:	11

The Filter Equation:	11

The Hardware Implementation:	11

1.	The Frequency Response	13

Simulation Diagrams:	13

The Output Result:	13

SNR:	13

Stage 3 – Cascaded Integrator-Comb:	14

1.	The CIC Filter Overview	14

2.	The Hardware Implementation	14

Simulation Diagrams:	16

SNR	18

The System Wrapper:	19

Register File Content:	19

Registers Mapping:	19

Fixed Point converting:	19

Table of Accumulator Bits:	20

Waveforms output	21

Table of figures

Figure 1 - System block-level	4

Figure 2:System block-level	4

Figure 3:Fractional Decimator Operation	6

Figure 4 - Fractional Decimator Operation	6

Figure 5 - Up sampling by factor of 2	6

Figure 6 - Down sampling by factor of 3	6

Figure 7 - Transposed Polyphase Decimator	7

Figure 8 - Fractional Decimator Python model output	8

Figure 9 - IIR Notch Filter Block-Level	9

Figure 10 - Direct Form II implementation	9

Figure 11 - One stage Notch filter using Hardware	10

Figure 12 - Notch Filter Frequency Response	11

Figure 13 – Notch Filter Python Model Output	11

Figure 14 - CIC Block-level	12

Figure 15 - CIC Architecture	12

Figure 16:Fracional Decimator Waveform	19

Figure 17:IIR Waveform	19

Figure 18:CIC Waveform	19

# Top Level DFE Overview:

Figure 1 - System block-level

Figure 2:System block-level

In modern digital communication and signal processing systems, the Digital Front-End (DFE) is responsible for preparing signals for further baseband processing. It typically performs sampling rate conversion, interference suppression, and spectral shaping.

The system receives the input signal with Fs=9 MHz. The fractional decimator works on down sampling to Fs=6 MHz, and then the notch filter supresses unwanted narrowband interference. Finally the CIC decimator reduces the sampling rate from Fs=6 MHz to a much lower rate Fs=6D MHz.

# System IO Table:

## DFE IOs:

| Signal name | Direction | Description |
| --- | --- | --- |
| clk | Input | System clock |
| rst_n | Input | Asynchronous active-low reset |
| notch_enable | Input | Active-low bypassing for notch filter |
| cic_decimation_factor [4:0] | input | Average filter decimation factor |
| data_in [width-1 : 0] | input | Input data |
| data_out [width-1 : 0] | output | Output data |

## AMBA APB IOs:

| Signal name | Direction | Description |
| --- | --- | --- |
| PCLK | Input | System clock |
| PRESETn | Input | Asynchronous active-low reset |
| PSEL | Input | Select. The APB bridge unit generates this signal to each peripheral bus slave. |
| PENABLE | input | Enable. This signal indicates the second and subsequent cycles of an APB transfer. |
| PWRITE | input | Direction. This signal indicates an APB write access when HIGH and an APB read access when LOW |
| PADDER [ADDR_WIDTH-1 : 0] | output | Address. This is the APB address bus. It can be up to 32 bits wide and is driven by the peripheral bus bridge unit. |
| PWDATA [DATA_WIDTH-1 : 0] |  | Write data. This bus is driven by the peripheral bus bridge unit during write cycles when PWRITE is HIGH. |
| PRDATA [DATA_WIDTH-1 : 0] |  | Read Data. The selected slave drives this bus during read cycles when PWRITE is LOW. |
| DFE Signals |  |  |
| input_data [DFE_WIDTH-1 : 0] |  | Input data of the DFE |
| output_data [DFE_WIDTH-1 : 0] |  | Output data of the DFE |

# Stage 1 - Fractional Decimator:

Convert the input signal, from 9 MHz ,to 6 MHz using fractional resampling. This is achieved using the L and M  LM= 23).

Figure 3:Fractional Decimator Operation

Figure 4 - Fractional Decimator Operation

The Fractional Decimator Overview:

First, the input signal will be up-sampled by a factor of 2 (Fs = 18 MHz) and then removed of imaging due to up-sampling using FIR filter, and finally it is decimated by a factor of 3 to reduce the sampling rate (Fs = 6 MHz).

Upsampling by 2: increasing the sampling rate of a discrete-time signal, x[n], by a factor of L=2, utilizing zero insertion.

Figure 5 - Up sampling by factor of 2

Figure 6 - Down sampling by factor of 3

Low pass FIR filter: is used before decimation for preventing aliasing and defining the new signal bandwidth, which is ≈ 3 MHz (new Nyquist).

Down sampling by 3: reduces the sampling rate by keeping one out of every 3 samples: yn=x[3n].

Figure 7 - Transposed Polyphase Decimator

Hardware Implementation:

The architecture is based on a Transposed Polyphase Filter.

For decimation by 3, we split the filter into 3 polyphase branches:

E0n=h3n

E1n=h3n+1

E2n=h[3n+2]

MUX Blocks: each MUX selects one tap from its polyphase group.

2-bit Counter: selects the correct coefficient for each cycle.

Multiplier Blocks: each MUX output is multiplied by the current input sample x(n).

Delay Chain: This delay structure stores partial FIR sums. In a transposed implementation, each register holds partial results for future outputs. The delay chain corresponds to tap delays, shared across cycles.

Adder Block: Each multiplier output is added to a stored partial sum. Over 3 cycles, each partial result accumulates all FIR contributions for one output sample. After the last cycle, the final output sample is complete.

Output Selector: The decimator outputs one sample every 3 cycles.

A modulo-3 counter enables output only when: cycle = 2 (end of polyphase accumulation). This sample is then labelled y(m).

## Simulation Diagrams:

Figure 8 - Fractional Decimator Python model output

Output Results:

SNR: The fractional decimator shows moderate attenuation, which is expected due to normalized polyphase filter coefficients. The required accumulator width is small, indicating limited dynamic range growth. The SNR of 75.50 dB reflects excellent accuracy and very low quantization noise.

# Stage 2 – Notch Filter:

Suppresses unwanted narrowband interferences at 5 MHz and 2.4 MHz relative to 6 MHz (after the first stage – fractional decimator). It’s resolved that the component of 5 MHz will be folded at 1 MHz due to aliasing and the filter can resolve frequencies up to 3 MHz.

Figure 9 - IIR Notch Filter Block-Level

Notch Filter Overview:

The input signal passes through two stages notch filter to remove the interference.

## Filter’s Equation:

Hz= 1-2rzcoswoz-1+ rz2 z-21-2rpcoswoz-1+ rp2 z-2

To achieve the BW <50 KHz and notch depth >50 dB, zero radius must be a little less than 1, and the pole radius too, to achieve stability.

Figure 10 - Direct Form II implementation

Hardware Implementation:

The filter can be written as:

yn=B0 xn+ B1 xn-1+ B2 xn-2- A1 yn-1- A2 y[n-2

Figure 11 - One stage Notch filter using Hardware

Notice that, the system is LTI with memory, so values need to be stored in register through the computation.

Where B0 , B1 , B2 , A1 , and A2 are the filter coefficients, computed using the Python model using S16.14 fixed point format.

| Sign | Integer | Fraction |
| --- | --- | --- |
| 1-bit | 1-bit | 14-bit |

With range [-1.999: 1.999].

| Coefficient | Floating point value | Fixed point value |
| --- | --- | --- |
| fo=1 MHz |  |  |
| B0 | 1 | 16’h4000 |
| B1 | -0.9999 | 16’hC000 |
| B2 | 0.9999 | 16’h4000 |
| A1 | -0.970 | 16’hC1EC |
| A2 | 0.9409 | 16’h3C38 |
| fo=2.4 MHz |  |  |
| B0 | 1 | 16’h4000 |
| B1 | 1.6180137633 | 16’h678E |
| B2 | 0.999972 | 16’h4000 |
| A1 | 1.569492 | 16’h6473 |
| A2 | 0.9409 | 16’h3C38 |

Figure 12 - Notch Filter Frequency Response

The Frequency Response:

BW ≈58 Hz.

The notch depth >50 dB.

## Simulation Diagrams:

Figure 13 – Notch Filter Python Model Output

The Output Result:

SNR: The IIR stage introduces additional attenuation, reducing signal amplitude further. An 18-bit accumulator is sufficient, but the SNR drops to 58.80 dB, which is typical for fixed-point IIR filters due to feedback sensitivity and coefficient quantization.

# Stage 3 – Cascaded Integrator-Comb:

The third stage of the Digital Front End (DFE) reduces the sample rate from 6 MHz to 6D MHz using a Cascaded Integrator-Comb (CIC) decimator with a runtime-selectable decimation factor D∈{1, 2, 4, 8, 16}.Fu

nctionally, the CIC stage:

- Produces 1 output for every D input, so the output sample is down sampled.

- Minimizes hardware cost (multiplier-free decimation), with fixed latency.

Figure 14 - CIC Block-level

The CIC Filter Overview:

The single-stage CIC first integrates the input by adding the current sampled input and the delayed sampled input, then decimates according to the selected value, and lastly, it differentiates the last two samples.

Figure 15 - CIC Architecture

The Hardware Implementation:

Since we decimate the sampling frequency, aliasing must occur, but it is a trade-off between the number of stages and the droop that happens.

If we increase the number of stages, the filter becomes more anti-aliasing and image-rejection, but on the other side, the drooping happens that requires a compensation filter to equalize the drooping.

Thus, a sweet spot was chosen of 3 stages for the CIC filter, manipulating the position of the three integrators and differentiators, and leaving the down sampling at the end of the stage. This architecture is widely used.

Also, another thing to consider is that multiple stages give gain according to this equation:

Gain=DmaxN= 163=4096

Where Dmax is the maximum decimated value, which is 16.

N is the number of cascaded stages of CIC.

Thus, sizing of the output should be done by accounting for the gain:

Bitgrowth=ceilN· log216=12 bits

Then the internal width of our register should be:

Width=16+12=28 bits

## Simulation Diagrams:

Figure 15 - Output for D = 2

Figure 14 - Output for D = 1

The Output Results:

Figure 17 - Output for D = 8

Figure 16 - Output for D = 4

Figure 18 - Output for D = 16

SNR: The CIC filter exhibits a gain close to what is expected from its cascade structure. The 30-bit accumulator is necessary to safely handle integrator growth without overflow. The SNR of 59.41 dB aligns with typical CIC behavior, where quantization noise and passband droop reduce accuracy compared to FIR-based stages.

# The System Wrapper:

Figure 19 - The Whole System with APB Interface

The system is wrapped with an APB interface to communicate with the microprocessor, and adjusting the system control signals or coefficients.

Register File Content: eight registers with width size 32-bit.

| Register | Content |
| --- | --- |
| R0 | Control signals |
| R1 | Status flags |
| R2 to R6 | Notch filter coefficients |

| R0 | Enable |  |  |  |  | Bypass |  |  |  | CIC factor |  |  | [12:3] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [0] | [1] | [2] | [3] | [4] | [5] | [6] | [7] | [8] | [9] | [10] | [11] | 1’d0 |
|  | FD | F01 | F02 | CIC | clk_div | FD | F01 | F02 | CIC | Factor [2:0] |  |  | Not used |

## Registers Mapping:

| Register | [0:17] | [18:31] |
| --- | --- | --- |
| R2 | B0_f01 | B1_f01 |
| R3 | B2_f01 | A1_f01 |
| R4 | A2_f01 | B0_f02 |
| R5 | B1_f02 | B2_f02 |
| R6 | A1_f02 | A2_f02 |

## Fixed Point converting:

The notch filter coefficient is represented by S16.14 (due to the coefficient range), and the data output from the fractional decimator is represented by S16.15. Between the fractional decimator and the notch filter, there is a S16.15 to S16.14 converter.

## Table of Accumulator Bits:

| STAGE | Accumulator Bits |
| --- | --- |
| Fractional Decimator | 18 bits |
| IIR filter | 18 bits |
| CIC | 30 bits |

# Waveforms output

Figure 16:Fracional Decimator Waveform

Figure 17:IIR Waveform

Figure 18:CIC Waveform

FPGA Flow:

Power consumption

STA

Source synced to the slowed clock

Resource Utilizaion